Use certain commands to modify fields in the SMBIOS.CFG file to write MT, PN, SN, UUID to the main board.

- Power on the computer and immediately press F2 to enter the BIOS Setup, modify Secure Boot to Disabled, and save the modification and restart the machine.

- Open Test Mode：

Open CMD as administrators, type the following command and then restart the machine.

# bcdedit /set testsigning on

- Copy the BIOS tool file to IdeaPad 4G or IdeaPad 5G desktop, open the file, and open CMD as administrators under this path.

- Type the following command to let the system read the SMBIOS.CFG file.

# QCFactoryTool.exe /RawFS SMBIOS GetFileData SMBIOS\SMBIOS.CFG SMBIOS.CFG 0

- Modify fields in the SMBIOS.CFG file. Open the SMBIOS.CFG file by NotePad, find the fields that need modify, for example,

Product Name: write,pn, "INVALID"

modify the strings in double quotation marks and save the change. For example, write PN ‘123’ to the main board, you can modify the fields to:

Product Name: write,pn, "123"

Note: It must have strings in double quotation marks. If there is nothing in the double quotation, type the Space tab. The following is an example of this note:

Right: write,pn," "

Wrong: write,pn,""

- Type the following command to let the system write the SMBIOS.CFG file.

# QCFactoryTool.exe /SMBIOS SMBIOS.CFG

- Restart the machine.